# SPECIAL PROVISIONS

FOR THE

TOWN OF FLORENCE PROJECT

FLORENCE ARIZONA, PINAL COUNTY

HUNT HIGHWAY AT ATTAWAY ROAD

# INTERSECTION IMPROVMENTS

PROPOSED WORK:

The proposed project is located in Pinal County at the intersection of Hunt Highway and Attaway Road. Attaway Road crosses over the Bureau of Indian Affairs San Carlos Irrigation Project (BIA SCIP) North Side Canal in Florence. The proposed work consists of widening the existing asphalt pavement at the intersection to provide dedicated left and right turn lanes, removing and replacing the asphaltic concrete (AC) pavement, mill and overly the existing travel lanes, modifications to the existing traffic signal, signing and striping, install guardrail and end treatments, and other related work.

# Specifications:

The work embraced herein shall be performed in accordance with the requirements of the following separate documents:

Arizona Department of Transportation, Standard Specifications for Road and Bridge Construction, Edition of 2021,

Arizona Department of Transportation, Roadway Engineering Group, Construction Standard Drawings, listed in the project plans, and available on ADOT’s website,

Arizona Department of Transportation, Traffic Group, Manual of Approved Signs, available on ADOT’s website,

Arizona Department of Transportation, Traffic Group, Traffic Control Design Guidelines, 2019 Edition, available on ADOT’s website,

Manual on Uniform Traffic Control Devices for Streets and Highways, 2009 edition and Arizona Supplement to the 2009 edition, dated January 2012,

Maricopa Association of Governments (MAG) Uniform Standard Specifications and Details for Public Works (latest edition and revisions),

American Association of State Highway and Transportation Officials (AASHTO) A Policy on Geometric Design of Highways and Streets (2018),

AASHTO Roadside Design Guidelines (2011),

AASHTO LRFD Bridge Design Specifications (8th Edition, 2017)

San Carlos Irrigation and Drainage District (SCIDD) Bridge Crossing Guidelines (February 2022), and

These Special Provisions.

# Environmental Mitigation Measures:

The project mitigation measures are not subject to change without the written approval from BIA SCIP.

Contractor Responsibilities

- To prevent the introduction of invasive species seeds, all earthmoving and hauling equipment shall be washed prior to entering the construction site and the contractor shall inspect all construction equipment and remove all attached debris, including plant parts, soil, and mud, prior to the equipment entering the construction site.

- To prevent invasive species seeds from leaving the site, the contractor shall inspect all construction and hauling equipment and remove all debris, including plant parts, soil, and mud, prior to leaving the construction site

- If vegetation clearing will occur during the migratory bird breeding season (March 1 to August 31), it is recommended that the contractor avoid any active bird nests. If active nests cannot be avoided, it is required that the permittee notify BIA to evaluate the situation. During the nonbreeding season (September 1 to February 28), vegetation removal is not subject to this recommendation.

- The contractor shall not cause injury or death to swallows, including eggs and nestlings. If work will occur that will directly impact nesting swallows from February 1 to August 31 of any calendar year, the contractor shall adhere to the following:

  - The contractor shall completely remove all inactive swallow nests within 100 feet of work areas after August 31 but prior to February 1 to prevent swallows from reusing those nests.

  - The contractor shall implement exclusionary measures to prevent swallows from building new nests within areas directly impacted by construction activities. Exclusionary measures shall be implemented in all areas where swallows are likely to nest and may include (a) continually removing nesting materials during early nest construction when eggs or nestlings are not present, (b) installing deterrent spike strips, and/or (c) installing polytetrafluorethylene (Teflon) sheeting.

  - The contractor shall not disturb any active swallow nests (completed or partially completed nests that contain eggs or nestlings). If any active nest is discovered within 100 feet of construction activities, work shall stop and the BIA shall evaluate the potential for disturbance of nests.

# Disposal of Existing Asphaltic Concrete:

Upon removal, disposal of the existing asphaltic concrete shall be the responsibility of the contractor.

# SECTION 104 SCOPE OF WORK:

104.04	Maintenance of Traffic: of the ADOT Standard Specifications is modified to add:

# (D)	General Traffic Control Information:

The project shall be constructed in accordance with Part VI of the Manual on Uniform Traffic Control Devices (MUTCD) 2009 Edition. Other references may include the Arizona Supplement to the 2009 MUTCD and the ADOT Temporary Traffic Control Design Guidelines (TCDG) 2019. The contractor is responsible for developing the traffic control plans necessary to provide a safe and efficient work zone without unduly delaying traffic. The Town or its representative shall approve the traffic control plans. Any changes or revisions to the approved traffic control plans must be approved in advance by the Town or Engineer.

The contractor shall provide and maintain all required and requested traffic control devices to protect and guide traffic for all work in the construction area. Intersection area shall be defined as all of the area within the right-of-way of intersecting streets, plus five hundred (500) feet beyond the center of the intersected streets on all legs of the intersection.

The construction schedule and associated traffic schemes shall be developed in such a manner that access is maintained at all times to adjacent businesses, residences, and emergency response vehicles, unless otherwise approved by the Town or Engineer. The schedules shall be developed in a format suitable for release to other agencies and the public and shall be updated, as necessary.

The contractor shall maintain all existing traffic signs erect, clean and in full view of the intended traffic at all times. Street name signs at major street intersections shall be maintained erect at all times. If any signs interfere with construction, the contractor shall notify the Engineer at least 48 hours in advance before covering or removing said signage. The contractor shall be responsible for reinstalling all signs removed or covered and verifying they are correctly placed. The

Town or Engineer will inspect all signage prior to completion of the project.

The temporary removal or covering of existing signs shall be in accordance with Section 701-3.11 and 701-5.10 of the Specifications. Traffic control devices will not be permitted to remain stockpiled on the shoulder in between closures, unless the contractor has approval from the Town or Engineer.

Signs installed on rigid sign stands or spring stands shall be mounted at a minimum height of 7 feet as measured from the bottom of the sign to the near edge of pavement. Signs shall not be installed on embedded posts.

The contractor will be required to check on all traffic control barricades, signs, and variable message boards twice daily and maintain as needed per the Subsection of the Specification 701-3.02 Maintenance and Protection of Traffic.

At all times, the contractor shall conduct the construction activities to safeguard pedestrians and vehicular access in the vicinity of the project.

(E) 	Schedules for Daily Traffic Control Activities:

The contractor shall provide a schedule of daily activities and corresponding traffic control schemes. The information shall be provided in advance of or during the weekly construction meeting in accordance with the requirements of Subsection 108.03 of the specifications and shall specify the limits of the work activities and related traffic control plan by location, direction, and time.

All travel lanes will be subject to the following:

- No full road closures are permitted unless approved by the Town or Engineer.

- No lane closures will be allowed during weekends and Holidays unless approved by the Town or Engineer

- The contractor shall always maintain vehicular access within the project limits, unless otherwise approved by the Town or Engineer.

Holidays

- The contractor shall not schedule a closure to begin prior to a holiday.

- Holidays are from 12:00 PM of the last working day prior to the holiday until 6:00 AM of the first working day following the holiday. When holiday falls on Tuesday, the Monday before the holiday is considered part of the holiday. When the holiday falls on a Thursday, the Friday following the holiday is considered part of the holiday.

- The recognized holidays are:

- Martin Luthur King Jr. Day

- President Day

- Memorial Day

- Independence Day

- Labor Day

- Columbus Day

- Veterans’ Day

- Thanksgiving

- Christmas Day

- New Years Day

(104SWDEQ, 02/10/20)

SECTION 104	SCOPE OF WORK:

104.09		Prevention of Stormwater Pollution: of the Standard Specifications is revised to read:

(A)		General:

The contractor shall follow the requirements of Section 402 of the Clean Water Act (CWA), 33 USC 1251 and other water quality regulations.  The CWA regulates discharge of pollutants into the waters of the United States and provides quality standards for surface waters.  Section 402 of the CWA provides stormwater pollution prevention regulations.  ADEQ’s Aquifer Protection Permit (APP) Program regulates temporary concrete washout facilities.  The APP Type 1 Permit (APP 1.12) applies only when projects disturb one or more acres and are covered under the Arizona Department of Environmental Quality Construction General Permit.

The contractor shall minimize ground disturbance to the natural surroundings.  No ground disturbing activities shall occur until all applicable permits have been obtained.  The applicable permit requirements must always be met.

The contractor shall provide adequate information to the contractor’s personnel, including employees of any subcontractors, to ensure that all personnel understand requirements of the applicable permits that are relevant to their job functions.

(B)		Clean Water Act, Section 402, Arizona Pollutant Discharge Elimination System (AZPDES):

(1)	General Requirements:

The contractor shall be responsible for preparing and processing all documents and implementing the requirements of the Arizona Pollutant Discharge Elimination System (AZPDES) “General Permit for Discharge from Construction Activities to the Waters of the United States,” for pollution prevention issued by the Arizona Department of Environmental Quality (ADEQ).  That document is hereinafter referred to as the Construction General Permit (CGP) and is available on ADEQ’s website.

The work shall include providing, installing, and maintaining, temporary and permanent pollution prevention control measures and removing and disposing of temporary pollution prevention control measures.  Control Measures, as hereinafter referenced, shall be deemed to include pollution prevention control as described in the CGP.  Control Measures may be temporary or permanent.

The contractor’s schedule shall allow sufficient time for the following submittal, review, and approval times:

| Table 104-1 Submittal, Review, and Approval Timeframes |  |
| --- | --- |
| Contractor submittal of Erosion Control Coordinator (ECC) qualifications | By the pre-construction conference |
| Department review and approval of ECC qualifications | within seven calendar days of receipt |
| Department review and approval of subsequent submittals (if required) | within seven calendar days of receipt |
| Contractor submittal of SWPPP to Engineer | By the pre-construction conference |
| Department review and approval of subsequent submittals (if required) | within seven calendar days of receipt |
| Engineer/contractor joint review of SWPPP | within 10 calendar days after contractor submittal of SWPPP with ADOT approval of the ECC |
| Approved Notice of Intent (NOI) submittal and review times | as described in the CGP |

No increase in contract time will be granted for the contractor’s failure to provide acceptable submittals within the specified timeframes.

(2)	Stormwater Pollution Prevention Plan (SWPPP):

The SWPPP shall include all information and permit requirements specified in the CGP, and shall also be consistent with applicable state or local programs.

Unless otherwise approved by the Engineer and documented in the SWPPP, the contractor shall not expose a surface area of greater than 750,000 square feet to erosion through clearing and grubbing, or excavation and filling operations within the project limits until temporary or permanent Control Measures for that portion of the project have been installed and accepted by the Engineer.

The contractor shall indicate each 750,000 square-foot sub-area in the SWPPP, along with proposed Control Measures for each sub-area.  The SWPPP shall also include the sequence of construction for each sub-area, and installation of the required temporary or permanent Control Measures.

The contractor shall give installation of permanent Control Measures priority over reliance on temporary measures.  Permanent Control Measures and drainage structures shall be installed as soon as possible in the construction sequencing of the project, preferably concurrent with construction of the related sub-area or drainage device.

The project plans may include erosion/sediment control plans and details along with a Control Measure Index Sheet (CMIS) to assist the contractor in preparing the SWPPP.  The contractor and the contractor’s ECC, or other qualified designee, as described in Subsection 104.09(B)(3), shall review the contract documents, and prepare the SWPPP for review by the Engineer.  The contractor shall use the Department’s SWPPP Template to prepare the SWPPP.  The template is available on the Department’s Water Resources website.

The SWPPP shall specify the mechanism whereby revisions may be proposed by the contractor or the Engineer throughout the project and incorporated into the plan, including the review and approval procedure.  The Engineer and contractor shall jointly approve and sign each revision to the SWPPP before implementation.  Any revisions made by the contractor to amend the approved SWPPP will require two to seven calendar days for review.

The contractor shall prepare the SWPPP for the Engineer’s review including all information specified herein.  The contractor shall submit the SWPPP to the Engineer at the preconstruction conference.

Within 10 calendar days of the SWPPP submittal, the Engineer and contractor will jointly review the contractor’s SWPPP. The contractor shall include any additional revisions directed by the Engineer.  The SWPPP shall meet the terms and conditions of the CGP and be compatible with construction sequencing and maintenance of traffic plans.

When agreement has been reached, the Engineer and contractor’s ECC will sign the SWPPP.  The Engineer's signature will constitute approval of the SWPPP.  Upon approval of the SWPPP, the contractor and the Engineer shall each file a Notice of Intent (NOI) as specified in Subsection 104.09(B)(6).  The Engineer will provide a copy of the Department’s NOI to the contractor to be included in the SWPPP.

The SWPPP shall be maintained in accordance with the CGP.  A current copy shall be maintained at a location approved by the Engineer and amended as necessary from the time the contractor files its NOI until the NOT is submitted.  The SWPPP shall be available for inspection by entities identified in the CGP, and for use by the Engineer.

No condition of the CGP or the SWPPP shall release the contractor from any responsibilities or requirements under other environmental statutes or regulations.

In accordance with the CGP, the Engineer and the contractor shall confirm the frequency of inspections appropriate for the project.  The contractor shall identify the frequency in the SWPPP.  The contractor shall inform the Engineer when each inspection will be performed a minimum of 72 hours in advance so the Engineer can jointly perform the inspection if desired.

The contractor shall monitor rainfall on the site with a commercially manufactured rain gauge accurate to within 0.10 inches of rain.  Rainfall records shall be maintained in the SWPPP.

(3)	Erosion Control Coordinator (ECC):

The contractor shall designate an erosion control coordinator (ECC) to be responsible for approval and implementation of the SWPPP, as well as all other applicable requirements of the CGP.  The contractor’s ECC shall have the qualifications described in Subsection 104.09(B)(4).

The contractor shall not assume that the person proposed as ECC will be accepted by the Department merely because the experience and education requirements listed herein have been met.

After approval, the contractor shall designate the ECC as a duly authorized representative of the contractor in accordance with, and as defined in, the CGP.  Documentation of the delegation of a duly authorized representative shall be included as part of the SWPPP.

The ECC shall be capable of identifying existing and predictable effects of the contractor's operations, and shall have complete authority to direct the contractor’s personnel and equipment to implement the requirements described herein, including prompt placement of corrective measures to minimize or eliminate pollution and damage to downstream watercourses. Corrective measures shall be completed in the timeframe required by the CGP and included in the SWPPP as required by the CGP.

The ECC shall at all times be aware of the contractor’s work activities, schedule, and effect of the work on the environment.  Should the ECC not be present at the project site on a full-time basis, the contractor shall establish procedures to ensure that its ECC is promptly notified of any damage or displacement of the required erosion control measures, whether from construction, vandalism, or other causes.

The contractor bears all risks and liabilities for the failure of its ECC to properly implement the requirements of the CGP.

Failure of the contractor to properly maintain the Control Measures required in the approved SWPPP may be cause for the Engineer to reject the ECC and issue a stop work order, as specified in Subsection 104.09(B)(7).

(4)		ECC Certification Requirements:

The ECC shall have successfully completed the mandatory two-day (16 hour) “Erosion Control Coordinator” training class (hereinafter referred to as the training class) provided by the Associated General Contractors (Arizona Chapter), phone (602) 252-3926.  No other training can be substituted.

The training class certification is valid for three years.  Within 60 days of the end of the three year period, in order to maintain the training class certification, the ECC shall successfully complete either a six-hour “Erosion Control Coordinator Refresher” class (hereinafter referred to as the refresher class), also provided by the Associated General Contractors (Arizona Chapter), or the two-day training class specified above.  The refresher class will be required every three years thereafter, prior to the expiration date listed on the previous certificate.  Should more than three years (and 60 day grace period) elapse from completion of either the training class or refresher class, the contractor’s ECC will be required to successfully complete the two-day training class in order to again be eligible for consideration.

In addition, the contractor’s ECC shall have documented experience equal to a minimum of one year from either of the following two categories:

(a)	Experience in the implementation of SWPPPs.  The ECC’s experience shall demonstrate full-time responsibility for directly supervising construction personnel in the installation, inspection, and maintenance of pollution prevention measures.

(b)	Experience in stabilization of disturbed areas in environments similar to those on the project.

The contractor’s documentation shall provide details indicating the types of relevant experience and shall provide the number of months of each type of experience to be considered for approval.

The contractor’s documentation shall also indicate that the proposed ECC has completed the training class or refresher class.  To be considered for approval, the contractor’s documentation shall include a copy of the proposed ECC’s certification.

(5)		Acceptance of ECC and SWPPP:

The contractor shall submit documentation indicating the qualifications of the ECC to the Engineer for approval no later than the pre-construction conference. The Engineer will review the proposed candidate’s information within seven calendar days.  The contractor may begin development of the SWPPP prior to approval of the ECC.  However no clearing, grubbing, earthwork, or other work elements that may be subject to the requirements of the CGP shall be started until the ECC has been approved, the SWPPP finalized and implemented, the NOI completed and filed, and the CGP authorization received by the contractor and the Engineer.  If sampling is required, the SWPPP and SAP shall also be reviewed and approved by ADEQ prior to ground disturbance by the contractor.

(6)		Notice of Intent:

After the project SWPPP has been approved by the Engineer, the Engineer and the contractor will each complete separate Notice of Intent (NOI) forms for the project.    The contractor shall submit the NOI to ADEQ as required by the CGP.

Unless notified otherwise by ADEQ, the contractor will be authorized to begin implementation of the approved SWPPP seven calendar days after acknowledgement of receipt of both NOIs by ADEQ or whenever an authorization certificate is issued by ADEQ, whichever occurs first.

(7)		Non-Compliance:

The Engineer may reject the contractor’s ECC if, in the opinion of the Engineer, the conditions of the CGP or the SWPPP are not being fulfilled.  Rejection of the contractor’s ECC shall be for failure to complete any of the following:

(a)	Should the Engineer determine that the SWPPP is not being properly implemented; the contractor will be notified in writing of such deficiencies.  The contractor’s ECC shall fully implement, to the satisfaction of the Engineer, the requirements of the approved SWPPP within three working days.

(b)	Should any corrective measures required in the CGP not be completed within the time periods specified therein, the Engineer will notify the contractor in writing.  The contractor’s ECC shall complete all required corrective measures within two calendar days of such notification, except that direct inflows of sediment into a watercourse shall be corrected within 24 hours.

(c)	Should the Engineer determine that routine maintenance of the project’s Control Measures is not being adequately performed; the contractor will be notified in writing.  Within three working days, the contractor’s ECC shall demonstrate, to the satisfaction of the Engineer that such steps have been taken to correct the problem.

In the event of the ECC’s failure to comply with the CGP or any of the above requirements, the Engineer will direct the contractor to stop all affected work and propose a new ECC as soon as possible.  However, all Control Measures specified in the SWPPP shall be maintained at all times.  No additional work on construction items affected by the SWPPP will be allowed until a new ECC has been approved by the Engineer.  The contractor will not be allowed compensation or an extension of contract time for any delays to the work because of the failure of the contractor’s ECC to properly fulfill the requirements of the SWPPP.

(8)		Notice of Termination:

Upon written approval of the Engineer, the contractor shall complete and submit a Notice of Termination (NOT) to the Engineer for approval.  After approval by the Engineer, the contractor shall submit the NOT to ADEQ.

The NOT shall be submitted as described in the CGP.

A copy of the SWPPP and the NOT shall be provided to the Engineer within seven days of the contractor receiving acknowledgement from ADEQ.  The copies may be either electronic or hard copies.

(C)		Aquifer Protection Requirements (Concrete Washout Facilities):

If the contractor’s work requires the use of temporary concrete washout facilities, the contractor shall comply with the requirements of ADEQ’s APP Type 1 Permit (APP 1.12).  APP 1.12 covers wastewater to an impoundment from washing concrete from trucks, pumps, and ancillary equipment.  The contractor shall address concrete washout activities in the project SWPPP.

(D)		Measurement and Payment:

Measurement and payment for work specified in the SWPPP will be made in accordance with the requirements of Section 810.  Control Measures specified in the contract which are to be accomplished under any of the other various contract items will be paid for as specified under those items.

If a force account pay item for Control Measures is included in the bidding schedule, the contractor may be reimbursed for such additional Control Measures proposed by the contractor but not included with the plans or specifications.  Such additional Control Measures must be approved in writing by the Engineer before use.  Items for Control Measures approved by the Engineer will be paid in accordance with Subsection 109.04(D).  No measurement or payment will be made for such additional items not approved by the Engineer.

No measurement or payment will be made to the contractor for time spent in preparing, reviewing, and revising the SWPPP, the SAP, or providing other required documentation, the cost being considered as included in the price of contract items.  No measurement or payment will be made for inspections, training of personnel, the contractor’s erosion control coordinator, the contractor's pollution prevention practices and requirements, or maintenance of the Control Measures during a suspension of work, the costs being considered as included in contract items.

No measurement or payment will be made, except as specified below for external laboratory testing, for labor, equipment, and materials required in the SAP, the cost being considered to be included in contract items.

If an item is included on the bidding schedule for Construction Monitoring (Laboratory Testing), the contractor will be reimbursed for those samples tested, as required in the approved SAP, by an accredited laboratory approved by the Arizona Department of Health Services.  The contractor will be reimbursed for the invoice amount of each required test, plus an additional markup of ten percent of the invoice amount.  No measurement or payment will be made for in-field testing and related equipment, the cost being considered to be included in contract items.

No measurement or payment will be made for temporary concrete washout facilities, the cost being considered to be included in contract items.

Unless otherwise specified, no measurement or payment will be made for maintenance of temporary and permanent Control Measures, the cost being considered as included in contract items.

104.10		Contractor's Responsibility for Work:  of the Standard Specifications is revised to read:

The contractor shall implement the requirements of the CGP for pollution prevention due to stormwater runoff during construction, as specified above in Subsection 104.09, Prevention of Stormwater Pollution.

Until final written acceptance of the project by the Engineer, the contractor shall have the charge and care thereof and shall take every precaution against injury or damage to any part thereof by the action of the elements, or from any other cause, whether arising from the execution or from the nonexecution of the work.  The contractor shall rebuild, repair, restore and make good all injuries or damages to any portion of the work occasioned by any of the above causes before final acceptance.  No reimbursement shall be made for work necessary due to the contractor's failure to comply with the requirements of the SWPPP.

Except as specifically provided under Subsection 104.04, in case of suspension of work from any cause whatever, the contractor shall be responsible for the project and shall take such precautions as may be necessary to prevent damage to the project and provide for normal drainage.  The contractor shall maintain Control Measures in working order during any stoppage of work.  Based on the nature of the work stoppage, the contractor and the Engineer shall determine the maintenance requirements.  The contractor shall comply with the CGP including inspection of the project.

(106DMAT, 02/15/11)

SECTION 106	CONTROL OF MATERIALS:

106.15		Blank: of the Standard Specifications is revised to read:

106.15		Domestic Materials and Products:

Steel and iron materials and products used on all projects shall comply with the current “Buy America” requirements of 23 CFR 635.410.

All manufacturing processes to produce steel and iron products used on this project shall occur in the United States.  Raw materials used in manufacturing the steel and iron products may be foreign or domestic.  Steel or iron not meeting these requirements may be used in products on this project provided that the invoiced cost to the contractor for such steel products incorporated into the work does not exceed either 0.1 percent of the total (final) contract cost or $2,500 whichever is greater.

Any process which involves the application of a coating to iron or steel shall occur in the United States.  These processes include epoxy coating, galvanizing, painting, or any other coating which protects or enhances the value of covered material.

The requirements specified herein shall only apply to steel and iron products permanently incorporated into the project.  “Buy America” provisions do not apply to temporary steel items, such as sheet piling, temporary bridges, steel scaffolding and falsework, or to materials which remain in place at the contractor’s convenience.

The contractor shall furnish the Engineer with Certificates of Compliance, conforming to the requirements of Subsection 106.05, which state that steel or iron products incorporated in the project meet the requirements specified.  Certificates of Compliance shall also certify that all manufacturing processes to produce steel or iron products, and any application of a coating to iron or steel, occurred in the United States.

Convict-produced materials may not be used unless the materials were produced prior to July 1, 1991 at a prison facility specifically producing convict-made materials for Federal-aid construction projects.

(106CMATLS, 12/21/23)

SECTION 106	CONTROL OF MATERIALS: of the Standard Specifications is modified to add:

106.17		Construction Materials:

A construction material, when used on a federal-aid construction project shall comply with the requirements of Build America, Buy America (BABA) Act specified in Title IX, Subtitle A, Part 1, Sections 70901 and 70911-70918 (Pub. L. No. 117-58 §§ 70901; §§ 70911-70918) of the Infrastructure Investment and Job Act (IIJA).

A “construction material” that is permanently incorporated on the project shall include an article, material, or supply that is or consists primarily of the following:

- Non-ferrous metals;

- Plastic and polymer-based products (including polyvinylchloride, composite building materials, and polymers used in fiber optic cables);

- Glass (including optic glass);

- Fiber optic cable (including drop cable);

- Optical fiber;

- Lumber;

- Engineered wood; or

- Drywall.

Items manufactured through a combination of either two or more materials listed above, or at least one of the materials listed above and a material not listed shall be considered as a manufactured product, rather than as a construction material.

Build America, Buy America provisions specified for manufactured products in Section 70912(6)(B) of the IIJA, do not apply to federal-aid construction projects per FHWA’s existing statutory requirement applicable to manufactured products. A “manufactured product” is considered to be an item that undergoes one or more manufacturing processes before the item can be used on a federal-aid construction project.

Construction materials shall not include cement and cementitious materials; bituminous materials; aggregates such as stone, sand, or gravel; or aggregate binding agents or additives.

All construction materials shall be produced in the United States. This means, all manufacturing processes to produce the construction materials shall occur in the United States. All manufacturing processes for construction materials shall mean the final manufacturing process and the immediately preceding manufacturing stage for the construction material.

The contractor shall furnish the Engineer with Certificates of Compliance, conforming to the requirements of Subsection 106.05 of the specifications, which shall state that the construction materials incorporated in the project meet the requirements specified herein.  Certificates of Compliance shall also certify that all manufacturing processes to produce construction materials occurred in the United States.

If the total cost of construction materials incorporated in the project is no more than 5% of the original contract amount or $1,000,000, whichever is lesser, the requirements specified herein will not apply for such construction materials.

Convict-produced materials are prohibited in accordance with the requirements of 23 CFR 635.417.

(107LPAINS, 08/02/19)

# SECTION 107 LEGAL RELATIONS AND RESPONSIBILITY TO THE PUBLIC:

107.15		Contractor's Responsibility for Utility Property and Services:  of the Standard Specifications is modified to add:

Copies of existing permits, subject to availability, may be obtained from the ADOT and Navajo County as listed below:

The following utility company have facilities in the area but are not anticipated to be in conflict:

| Company | Contact Person | Contact Number |
| --- | --- | --- |
| Arizona Public Service - Overhead Electric | Manuel Escarcega | (602) 371-7171 |
| Central Arizona Water Conservation District (CAP Canal) | Marcus Ratliff | (623) 869-2477 |

The following utilities have facilities in conflict which are to be relocated during construction of this project. The contractor shall coordinate his work to facilitate the work performed by the utility companies and to avoid delays or conflicts:

| Company | Contact Person/Number | Facilities Conflict/Location | Anticipated Completion Date |
| --- | --- | --- | --- |
| CTLQL – CenturyLink (1) | Kevin Wagner (815) 245-9640 | Coaxial and Fiber Optic/Underground and on Bridge (Northside) |  |
| Zayo Group, LLC | CFZayo.relo@cob bfendley.com zayo.relo.arizona @zayo | Fiber Optic/Underground and on Bridge (Northside) |  |
| Town of Florence | Timm Wainscott (520) 868-7619 | 6” Sewer Line (Abandoned)/ Underground and on Bridge (Northside) |  |
|  |  | 10” Water Line (PVC)/Underground and on Bridge (Northside) |  |
| Southwest Gas Corporation | Thomas Brown (520) 316-5019 | 4” High Pressure Gas Line/ Underground and on Bridge (Northside) |  |
| Arizona State Prison Florence Complex | Lilah Meyer (520) 709-5611 | 12” Effluent Line (Steel)/ Underground and on Bridge (Southside) |  |
|  |  | 12” Water Line (Steel)/Underground and on Bridge (Southside) |  |
|  |  | Fiber Optic/Underground and on Bridge (Northside) |  |
|  |  | 6” Water Line/Underground and on Bridge (Northside) |  |

The Contractor is cautioned to use care when operating near these facilities.

It shall be the contractor's responsibility to determine the exact location of the utilities prior to any construction operations and to notify the above utility companies at least two (2) working days prior to commencing any work on the project.

(108TIME, 10/15/20)

# SECTION 108	PROSECUTION AND PROGRESS:

108.08	Determination and Extension of Contract Time: the first paragraph of the ADOT Standard Specifications is revised to read:

The time allowed for the completion of the work included in the contract will be 150 calendar days and will be known as the "Contract Time."

(109RRBB, 04/18/24)

SECTION 109 	MEASUREMENT AND PAYMENT:

109.04(D)(3)		Equipment: the first paragraph of the Standard Specifications is revised to read:

Equipment which the Engineer considers necessary for the performance of work will be eligible for payment at the established rates only during the hours that it is operated except as otherwise allowed elsewhere in these specifications. Equipment hours will be recorded to the nearest one-half hour. For the use of equipment owned by the contractor and approved by the Engineer, the contractor will be paid the rental rates, as modified herein, outlined in the Rental Rate Blue Book® (RRBB) for Construction Equipment which is updated by EquipmentWatch™, a division of Fusable, 3200 Rice Mine Road NE, Tuscaloosa, Alabama 35406, phone (888) 307-1713. All rate determinations will be based on the RRBB quarterly rate revisions that are applicable at the time equipment is being used.

(109BITUMADJ, 04/15/21)

SECTION 109	MEASUREMENT AND PAYMENT:  of the Standard Specifications is modified to add:

109.16		Bituminous Price Adjustment:

- General:

The term "bituminous material" as used herein shall include asphalt binder, asphalt rubber and emulsified asphalt.

The Department will adjust monthly progress payments as appropriate for market fluctuations in the price of bituminous material.

A price for bituminous material will be determined monthly by the Department based on the selling prices published by the Asphalt Weekly Monitor, a publication of Poten & Partners, Inc.  The price will be the arithmetic average of the high and low selling prices for bituminous material shown in the previous four reports for the Arizona/Utah and Southern California regions.

This value will be effective as of the last Wednesday of each month, and will be posted on the ADOT Contracts and Specifications Group website, on or shortly after the last Wednesday of each month.

This price will be deemed the “initial cost” (IC) for bituminous material for projects on which bids are opened during the following month.  This price will also be deemed the "current price" (CP) for bituminous material for the following month for projects in construction.

- Measurement:

For each item of bituminous material for which there is a specific pay item, and for the bituminous material used in Asphaltic Concrete (Miscellaneous Structural), an adjustment will be made for each month that a quantity of bituminous material was used on the project.

The IC for the month in which the project was bid will be compared with the CP for the appropriate current month.  The CP will be as posted on the Department’s website on the last Wednesday of each month, and will be used to adjust costs for bituminous material incorporated into the job during the following month (for example; bituminous material used in May will be adjusted, as specified herein, based on the CP for May as posted on the last Wednesday of April).  Any difference in price between these two values will be applied to the quantity of eligible bituminous material incorporated into the work.

Determination of the eligible quantities of bituminous material for adjustment will be based on contractor-furnished invoices, except as described herein.

The tons of emulsified products to which the adjustment will be applicable will be the tons of the emulsified asphalt prior to dilution.

Adjustments in compensation for emulsified asphalts will be made at 60 percent of either the increase or decrease of IC.  For emulsified asphalts containing polymer, adjustments in compensation will be made at 66 percent of either the increase or decrease.

The tons of Bituminous Material (Asphalt-Rubber) to which the adjustment is applicable will be 80 percent of the total quantity of the item used.  The adjustment is not applicable to the 20 percent of the material which constitutes the crumb rubber additive.

The tons of bituminous material incorporated in Asphaltic Concrete (Miscellaneous Structural) or Asphaltic Concrete (Miscellaneous Structural-Special Mix) to which an adjustment will be applicable is as follows:

(1)	For mixes without reclaimed asphalt pavement (RAP), the adjustment will be equal to 5 percent of the quantity, measured in tons, of asphaltic concrete placed, regardless of the actual percentage of bituminous material incorporated into the mix.

(2)	For mixes with RAP, the adjustment will be equal to 4 percent of the quantity, measured in tons, of asphaltic concrete placed, regardless of the actual percentage of bituminous material incorporated into the mix.

(3)	If the quantity of asphaltic concrete is measured by volume, the supplemental agreement establishing the method of measurement will specify the manner in which the tons of bituminous material eligible for the adjustment is determined.

(C) 	Payment:

The tons of bituminous materials which are paid for on the basis of testing by nuclear asphalt content gauge, ignition furnace, or other approved methods to which the adjustment will be applicable, are the tons which have been incorporated into the mixture.

When RAP is used in asphaltic concrete, only the tons of virgin asphalt binder will be subject to a bituminous material price adjustment.  RAP binder will not be subject to a price adjustment.

No compensation will be made for changes that may have occurred since the time of bidding and which may be the result of any increase in the IC of bituminous material.

Adjustment in unit prices of items governed by this provision will be made in the next regular monthly progress payment following actual use or application of the bituminous material.

(201PAY, 07/15/21)

SECTION 201	CLEARING AND GRUBBING:

201-5			Basis of Payment:  of the Standard Specifications is modified to add:

When clearing and grubbing is not included as a contract pay item, full compensation for any clearing and grubbing necessary to perform the construction operations designated on the project plans or specified in the Special Provisions shall be considered as included in the price of contract item.

SECTION 202	REMOVAL OF STRUCTURES AND OBSTRUCTIONS: the first paragraph of the Standard Specifications is hereby deleted:

202-3.03 (B) 	Bituminous Pavement:  of the Standard Specifications is modified to add:

Asphaltic concrete is designated on the project plans to be removed and shall not be used for construction and shall become the property of the contractor.

ITEM 2020053 	REMOVE (POST):

Description:

The work under this item shall consist of removal of the existing post and its foundation at locations shown on the project plans.

Construction Requirements:

Removal of the post shall be in accordance with the requirements of Subsection 202-3.06 of the specifications.

Removal of the foundation shall be in accordance with the requirements of Subsection 202-3.04 of the specifications.

Method of Measurement:

Remove (Post) will be measured by the unit of each post and its foundation removed.

Basis of Payment:

The accepted quantities of remove (post), measured as provided above, will be paid for at the contract unit price each, which price shall be full compensation for the work, complete in place.

(203ERWK, 07/16/20)

SECTION 203	EARTHWORK:

203-9.02		Materials:  the last sentence of the Standard Specifications is revised to read:

Borrow placed within three feet of the finished subgrade elevation shall conform to the following requirement:

PC + (2.83 x PI) shall not exceed 87,

where:

PC	=	Percent of material passing the No. 200 sieve (determined in accordance with Arizona Test Method 201), and

PI	=	Plasticity Index (determined in accordance with AASHTO T 90).

(303RASBAB, 04/18/24)

SECTION 303	AGGREGATE SUBBASES AND AGGREGATE BASES:

303-2			Materials:  of the Standard Specifications is revised to read:

Aggregate for the various classes of aggregate subbases and aggregate bases shall consist of stone, gravel or other approved inert material of similar characteristics, and shall be clean and free from vegetable matter and other deleterious substances.

Aggregate subbase and aggregate base material may be comprised of processed salvaged asphaltic concrete, processed salvaged aggregate base material, processed salvaged Portland Cement Concrete materials, virgin aggregate base materials or any combination of these materials meeting the requirements of Table 303-1. These materials may be from a specific project site or approved commercial source. Processed salvaged materials shall conform to the requirements specified in AASHTO M 319.

Aggregate subbases and aggregate bases shall conform to the requirements of Table 303-1:

| TABLE 303-1 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class of Aggregate | Percent Passing Sieve (Inch or No.) |  |  |  |  |  |  |  |  | PI, Max. |
|  | 3 | 1-1/2 | 1 | 3/4 | 1/4 | 4 | 8 | 30 | 200 |  |
| 1 |  | 100 | 90 - 100 |  |  |  | 35 - 55 |  | 0 - 8.0 | 3 |
| 2 |  | 100 | 90 - 100 |  |  | 38 - 65 | 25 - 60 | 10 - 40 | 3.0 - 12.0 | 3 |
| 3 |  |  |  |  |  |  |  |  |  |  |
| 4 | 100 |  |  |  | 35 - 70 |  |  |  | 0 - 10.0 | 5 |
| 5 | 100 |  |  |  | 30 - 75 |  |  |  | 0 - 10.0 | 5 |
| 6 |  |  |  |  |  |  |  |  |  |  |
| Notes: - The PI (Plasticity Index) will be determined in accordance with the requirements of AASHTO R 58, T 89, and T 90. - Classes 1, 2 and 3 are bases; Classes 4, 5 and 6 are subbases. - The requirements for Class 3 and for Class 6 will be specified in the Special Provisions. - For Class 1 through Class 4 aggregate, the amount of one fractured face coarse aggregate particles shall be at least 50 percent. - Resistance to abrasion for Class 1 through Class 4 aggregate will be determined in accordance with the requirements of AASHTO T 96 and shall meet the following requirements: Maximum loss of 12 percent at 100 revolutions Maximum loss of 40 percent at 500 revolutions - When determining gradation of aggregate subbase or aggregate base material containing processed salvaged asphaltic concrete materials, drying to a constant weight shall be performed at a temperature of 140 ± 5 F, in accordance with the requirements of AASHTO T 265. |  |  |  |  |  |  |  |  |  |  |

Aggregate subbase and aggregate base material not conforming to the requirements of Table 303-1 for gradation and/or PI may be accepted by the Engineer upon the contractor’s submittal of testing results demonstrating that the R-Value is at least 79 when determined by AASHTO T 190. At the contractor’s option, the material may be reprocessed and re-compacted at no additional cost to the Department.

All metal reinforcement and expansion materials shall be removed from processed salvaged Portland Cement Concrete prior to its use in aggregate subbase and aggregate base material.

The contractor shall submit the testing information for the base and subbase materials which are intended to be used to the Engineer for approval. Any significant change in the materials shall be approved by the Engineer prior to use.

Mixing on grade shall be accomplished using a full depth reclamation machine or pulverizer, manufactured for this purpose.  Motor graders, gannon boxes, auger scrapers, or other similar devices will not be allowed for mechanical mixing on grade.

If suitable in-place aggregate subbase or aggregate base materials are available, the contractor shall have the option of re-using such materials as processed salvaged aggregate base.  Processed salvaged asphaltic concrete and salvaged Portland Cement Concrete materials may be blended with the processed salvaged aggregate subbase, processed salvaged aggregate base and virgin base aggregates.  The blended material shall be homogenous mixture in which there is no segregation, crusts, lumps, or nesting, and shall be sampled and tested to meet the requirements specified herein.

303-3.01		Placement: of the Standard Specifications is modified to add:

When processed salvaged materials are utilized, either from in-place or from a commercial source, the Engineer may require a test section of 1000 feet be placed utilizing the processed materials and construction methods proposed for use by the contractor. Full operations shall not proceed until the test section has been tested and approved by the Engineer. The contractor shall use the same equipment, material processing, and construction methods for the remainder of the construction, unless adjustments made by the contractor are approved in advance by the Engineer.

303-3.02		Compaction: of the Standard Specifications is revised to read:

Each layer of aggregate subbase and aggregate base material shall be compacted to a density of not less than 100 percent of the maximum density determined in accordance with the requirements of the applicable test methods of the ADOT Materials Testing Manual, as directed and approved by the Engineer.

The moisture content shall be determined per AASHTO T 265 when determining density for aggregate subbase and aggregate base material containing processed salvaged asphaltic concrete and/or processed salvaged Portland Cement Concrete.

When AASHTO T 265 is utilized to determine moisture content, acceptance testing results for density will be furnished to the contractor up to 24 hours after performance of in-place density testing to allow sufficient time for moisture determination.

303-5			  	Basis of Payment: of the Standard Specifications is revised to read:

The accepted quantities of aggregate subbase and aggregate base, measured as provided above, will be paid for at the contract unit price per cubic yard, which price shall be full compensation for the work, complete in place.

(409ACMS, 07/18/24)

SECTION 409	ASPHALTIC CONCRETE (MISCELLANEOUS STRUCTURAL):

409-2			Materials:  of the Standard Specifications is modified to add:

The bidding schedule quantity of asphaltic concrete is based on an estimated unit weight of 147 pounds per cubic foot.

409-2.02		Bituminous Material:  the first paragraph of the Standard Specifications is revised to read:

Asphalt cement shall be a performance grade (PG) asphalt binder, conforming to the requirements of Section 1005 of the specifications.  The type of asphalt binder shall be PG 76-16.

409-3.01		General:  the ninth paragraph of the Standard Specifications is revised to read:

All wheels and tires of compactors shall be wetted with water, or if necessary soapy water, or a release agent in order to prevent the sticking of asphaltic concrete. All other equipment surfaces shall be treated when necessary with a release agent. Only release agents evaluated through AASHTO Product Evaluation & Audit Solutions (formerly NTPEP) are acceptable for use. The results from AASHTO Product Evaluation & Audit Solutions (formerly NTPEP) testing, when tested in accordance with AASHTO T 383, shall conform to the requirements shown in the table below.

409-3.01		General:  the second row of the “Release Agent Test – Requirement” table of the Standard Specifications is hereby deleted:

# SECTION 409	ASPHALTIC CONCRETE (MISCELLANEOUS STRUCTURAL):

409-2	Materials:  of the ADOT Standard Specifications is modified to add:

The bidding schedule quantity of asphaltic concrete is based on an estimated unit weight of 147 pounds per cubic foot.

409-2.02	Bituminous  Material:	the first paragraph of the ADOT Standard Specifications is revised to read:

Asphalt cement shall be a performance grade (PG) asphalt binder, conforming to the requirements of Section 1005 of the specifications. The type of asphalt binder shall be PG 70-10.

# ITEM 6080120	SIGN (RELOCATE):

Description:

The work under this item shall consist of furnishing all materials, personnel, and equipment necessary for relocating signs in accordance with the details shown on the plans and the requirements of these special provisions.

Construction Requirements:

The contractor shall remove the sign panels, the existing sign posts and the foundations in accordance with the requirements specified in Section 202 of the foundations.

Upon removal, the existing sign posts and the foundations shall become the property of the contractor.

The removed sign panels shall then be installed on the new signposts at the locations shown on the plans and in accordance with the requirements specified in Subsection 608-3.02 of the specifications.

Any fasteners or hardware required to relocate the sign panels shall be furnished by the contractor at no additional cost to the Department.

During relocation, if the sign panel is damaged due to contractor’s operations, the damaged sign panels shall be replaced or repaired to the satisfaction of the Engineer at no additional cost to the Department.

Method of Measurement:

Sign (Relocate) will be measured as a unit for each sign panel relocated.

Basis of Payment:

The accepted quantities of Sign (Relocate), measured as provided above, will be paid for at the contract unit price each, which price shall be full compensation for the work, complete in place.

Sign Posts and the Sign foundations at the new locations will be paid under the respective items.

(701LUMPSUM, 07/21/22)

SECTION 701 	MAINTENANCE AND PROTECTION OF TRAFFIC:

701-1			Description:  the first paragraph of the Standard Specifications is revised to read:

The work under this section shall consist of providing flagging services, and furnishing, installing, maintaining, moving and removing barricades, warning signs, lights, signals, cones, temporary pavement markings and other traffic control devices to provide safe and efficient passage through and/or around the work and to protect workers in or adjacent to the work zone.  The work shall be done in accordance with the requirements of Part 6 of the Manual on Uniform Traffic Control Devices (MUTCD) and the associated Arizona Department of Transportation supplement.  When referred to herein, these documents will be referred to as MUTCD and associated ADOT Supplement.

701-1			Description:  the third paragraph of the Standard Specifications is revised to read:

Any traffic control plan included in the project plans shall govern unless an alternate plan, acceptable to the Engineer, is submitted by the contractor.  If no traffic control plan is provided or if the contractor desires to deviate from the provisions for maintaining traffic as described in this section, it shall submit to the Engineer for the approval of a proposed sequence of operations and a compatible method of maintaining traffic.

701-3.01			General: of the Standard Specifications is modified to add:

At the preconstruction conference the contractor shall submit a log of all existing signs within the limits of the construction zone.  The Department will verify the accuracy of the sign log.  Throughout the duration of this project, all existing traffic signs shall be maintained by the contractor in accordance with Subsection 701-3.11 of the specifications.

The contractor shall coordinate all traffic control work on this project with any adjoining or overlapping projects during the construction activities.  This may include barricade placement necessary to provide a uniform traffic progress.

701-4.01		General:  the first paragraph of the Standard Specifications is revised to read:

The Department will reimburse the contractor for the work of maintaining and protecting traffic on a lump sum basis except for the following Elements of Work:

- Changeable Message Boards

- Temporary Impact Attenuation Devices

- Temporary Concrete Barrier

- Specialty Signs

- Law Enforcement Officers

- Flaggers

- Pilot Vehicles

- Truck-Mounted and Trailer-Mounted Attenuators

No additional measurement for payment to the contractor will be made for any Elements of Work other than those listed above.

701-4.01	General:  the first sentence of the third paragraph of the Standard Specifications is revised to read:

Elements of Work except those reimbursed under the lump sum item will be measured for payment as follows:

701-4.03(E)	Limitation of Measurement:  the second paragraph of the Standard Specifications is revised to read:

For all items except those reimbursed under the lump sum item, measurement will be made after the initial installation and once weekly thereafter for items in continuous use and at any other times changes are made in the use of traffic control elements listed under Subsection 701-4.01(B) of the specifications.  The contractor shall notify the Engineer when any changes are made in the use or location of traffic control elements.

701-4.04		Measurement of Work Elements:  Sub-paragraphs (D), (G), (H), (J), (K), (L), and (N) of the Standard Specifications are hereby deleted.

701-5			Basis of Payment for Elements of Work (Complete–in-Place): of the Standard Specifications is modified to add:

All elements of work necessary for the work of maintaining and protecting traffic, excluding Elements of Work listed in Subsection 701-4.01 of the specifications, shall be reimbursed as a single, complete item of work, which price shall be considered full compensation for the work, complete in place, including furnishing, removing, setting up, relocating, taking down, maintaining, and storing the devices.

Lump sum traffic control will be paid according to the schedule shown in Table 701-5 of the specifications.

| Table 701-5 Payment Schedule for Lump Sum Traffic Control |  |
| --- | --- |
| Percent of Total Contract Amount Earned (Excluding Mobilization and Traffic Control) | Allowable Percent of Lump Sum Price for Maintenance and Protection of Traffic |
| 5 | 20 |
| 25 | 40 |
| 50 | 60 |
| 75 | 80 |
| 90* | 100 |
| *Or substantial completion |  |

If the Engineer determines that the final contract amount earned will fall between 75 and 100 percent of the original contract amount, lump sum traffic control will be compensated at 100 percent once final acceptance is achieved.

In the event the Department revises the contract in accordance with Subsection 104.02 of the specifications due to a significant change in the character of work, and the revisions impact the Maintenance and Protection of Traffic, the lump sum price for Maintenance and Protection of Traffic will be adjusted as part of the contract revision under the relevant supplemental agreements.

No adjustments to the lump sum price for Maintenance and Protection of Traffic will be made for suspension of work for seasonal restrictions.

Revisions in the phasing of construction or maintenance of traffic control operations, requested by the contractor, may require traffic control to be installed in accordance with standards and/or designs other than those included in the plans.  Such revisions or modifications to the traffic control plans shown in the contract shall be submitted by the contractor for approval by the Engineer. No additional payment will be made for the revised plans requested by the contractor.

701-5.03 		Temporary Preformed Markings for Pavement:  of the Standard Specifications is hereby deleted.

701-5.05 		Obliterate Pavement Marking:  of the Standard Specifications is hereby deleted.

701-5.06 		Temporary Pavement Markers and Chip Seal Pavement Markers:  of the Standard Specifications is hereby deleted.

701-5.07 		Obliterate Pavement Markers:  of the Standard Specifications is hereby deleted.

701-5.08 		Temporary Delineators:  of the Standard Specifications is hereby deleted.

701-5.10		Temporary Removal or Covering of Signs:  of the Standard Specifications is hereby deleted.

701-6.01		Quantity Variances:  of the Standard Specifications is revised to read:

Payment for variances in quantities of items listed in Subsection 701-4.01 of the specifications shall be in accordance with Subsection 104.02 of the specifications, except that, for decreases in quantities, Temporary Concrete Barrier (In-Use) will be considered as a major item.

701-6.03		Channelization Devices:	of the Standard Specifications is hereby deleted.

701-6.04		Temporary Signs:  of the Standard Specifications is hereby deleted.

701-6.06		Flashing-Arrow Panels, and Changeable Message Boards:  the title and text of the Standard Specifications are revised to read:

701-6.06		Changeable Message Boards:

The accepted quantities of changeable message boards, measured as provided above, will be paid for at the unit bid price per day, which price shall be full compensation for the work, complete, including any grading and furnishing, operating, maintaining, and relocating the boards on the work site.

ITEM 9080103 	CONCRETE CURB AND GUTTER, TYPE C (MAG DET. 220-1):

Description:

The work under this item shall consist of furnishing all materials and constructing Concrete Curb and Gutter (Type C, MAG 220-1) at the locations shown on the project plans in according with the details shown on the plans and the requirements of these specifications. The transition sections of the Curb and Gutter is incidental to and included as part of this bid item.

Materials:

The materials shall conform to section 908 of the specifications.

Construction Requirements:

Concrete curb and gutter shall be constructed in accordance with the requirements specified in MAG Detail 220-1.

Method of Measurement:

Concrete Curb and Gutter (Type C, MAG DTL 220-1) will be measured by the linear foot along the flow line. Concrete Curb and Gutter 5-foot transitions from Type C to Type B or Type B to Type C will be measured as Type C Concrete Curb and Gutter.

Basis of Payment:

The accepted quantities of concrete curb and gutter (Type C, MAG DTL 220-1), measured as provided above, will be paid for at the contract unit price per linear foot, which price shall be full compensation for the work, complete in place, including furnishing and placing embankment material, excavating, removing unsuitable material, backfilling, compacting and surface finishing.

(925SRVY, 08/16/19)

SECTION 925	CONSTRUCTION SURVEYING AND LAYOUT:

925-5	Basis of Payment: the fourth paragraph of the Standard Specifications is revised to read:

If additional staking and layout are required as a result of additional work ordered by the Engineer, such work will be paid under items listed in the table below.

| ITEM | PREDETERMINED RATE |
| --- | --- |
| 9250101-ONE-PERSON SURVEY PARTY | $110 per hour |
| 9250102-TWO-PERSON SURVEY PARTY | $150 per hour |
| 9250103-THREE-PERSON SURVEY PARTY | $190 per hour |
| 9250106-SURVEY MANAGER | $175 per hour |
| 9250105–OFFICE SURVEY TECHNICIAN | $85 per hour |

(1001MATL, 06/17/21)

SECTION 1001 	MATERIAL SOURCES: of the Standard Specifications is revised to read:

1001-1	Description:

The work under this section shall consist of the procuring of borrow, topsoil, subbase and base materials, mineral aggregates for concrete structures, surfacing, and landscape plating, from sources either designated on the project plans or in the Special Provisions or from other sources.

1001-2	General:

The contractor shall determine for itself the type of equipment and work required to produce a material meeting the requirements of the specifications.

Sites from which material has been removed shall, upon completion of the work, be left in a neat and presentable condition. Where practicable, borrow pits, gravel pits, and quarry sites shall be located so that they will not be visible from the highway.

The contractor shall provide an environmental analysis, as specified in Subsection 104.12 of the specifications, for any source proposed for use regardless of whether a previously approved environmental analysis exists for the site.

In accordance with Subsection 104.12 of the specifications, the contractor may utilize an existing environmental analysis approved after January 1, 1999, provided that the analysis is updated as necessary to be in compliance with current regulations and with the contractor’s planned activities.

It shall be the responsibility of the contractor to conduct any necessary investigations, explorations, and research, on-site and otherwise, before and after submitting the bid proposal, to satisfy itself that the specified quantity and/or quality of material exists in any proposed material source.

The contractor shall not produce material for the project, mobilize crushing equipment or clear a worksite prior to approval of the environmental analysis.

The contractor shall comply with the requirements of the landowner or agency having jurisdiction over the land.

The Department makes no representation regarding quality or quantity of materials in any source.

1001-2.01	Information Available:

The Department maintains a listing of materials sources for which a completed environmental analysis is available and the landowner has allowed the source to be placed on the list.  Further information on material sources is available at https://azdot.gov/business/environmental-planning/material-source-guidance.

1001-2.02	Material Sources in Floodplains:

Any development of a material source that is determined to be in a flood plain must meet the requirements of the appropriate local, state, and federal agencies, including as applicable, the U.S. Army Corps of Engineers, Section 404 of the Clean Water Act, ADEQ or Tribal 401 Water Quality Certification, and the National or Arizona Pollutant Discharge Elimination System (NPDES/AZPDES).

If the contractor wishes to procure a material source within a floodplain, the contractor or material supplier shall submit a Floodplain Use Permit application to the appropriate floodplain management agency. The contractor shall submit to the Department documentation that the Floodplain Use Permit for the material source was approved and signed by the appropriate agency’s Floodplain Administrator.  The contractor or material supplier shall comply with all the requirements of the Floodplain Use Permit, including renewal of the Permit as needed or required.

The Department will require an engineering report if the material source is situated in the 100 year floodplain of any stream or watercourse, and located within one mile upstream and two miles downstream of any highway structure or surfaced roadway crossing.  The engineering report shall be prepared by a professional engineer with expertise in hydrology, hydraulics, river mechanics, and fluvial geomorphology.  The engineering report shall address the effects of the potential for structural damages following a 100 year flood event.

All other permits required to obtain a material source shall be furnished to the Department upon request.

Surplus material from agency administered flood control management projects may be used as borrow material only if the contractor submits written evidence to the Engineer that the flood control agency project was fully designed and funded.

Material sources in floodplains located on Native American Tribal Lands will be considered for use on an individual analysis.  The analysis shall include a review of applicable land use plans, floodplain management plans, environmental plans, applicable laws and regulations pertaining to Native American Tribal Lands, and an engineering analysis of the effects on any highway facility or structure.  The contractor shall obtain from the Bureau of Indian Affairs (BIA) and the Native American Tribal Council all permits, licenses, and approvals for the Department to review.

1001-2.03	Protection and Restoration:

The contractor shall comply with the requirements of Subsections 107.11 and 107.12 of the specifications for protecting and restoring the material source. The contractor shall assume full responsibility to protect and rehabilitate the material source to the satisfaction of the Department and in compliance with the requirements of the Federal Land Management Agency (FLMA) having jurisdiction or by the owner of the material source.

1001-3 	Proposed Source:

1001-3.01	Approval Requirements:

- General:

The contractor shall notify the Engineer prior to or at the preconstruction conference as to the source that it proposes to use.

The contractor acknowledges that all the conditions set forth in this subsection shall be met prior to the source being approved for use.

Other than sampling and testing, the requirements of this subsection shall be completed prior to initiation of any activities that disturb the existing conditions at the proposed source.

Regulatory changes, specification changes, or other reasons may preclude the approval of a materials source. The contractor acknowledges that the Department may refuse to approve a material source even if the Department had approved the source for other projects.

- Specific Conditions for Approval:

The use of a source will require written approval by the Engineer. No approval will be given until the contractor has complied with the following conditions:

- The contractor has submitted an environmental analysis, as specified in Subsection 104.12 of the specifications, of the source proposed for use and the Department has reviewed the analysis and satisfied itself that the use of such source will not have an adverse social, economic or environmental impact. The requirements of Subsection 1001-3.01 of the specifications shall be completed prior to initiation of any activities that disturb the existing conditions at the proposed source.

- The contractor has furnished the Engineer with evidence that he has secured the rights to the source, including ingress and egress.

- Historical and Cultural Resources:

The archaeological survey report of the proposed material source shall be prepared by the contractor’s archaeological consultant and shall conform to the requirements of Subsection 104.12 of the specifications.

In the event that prehistoric or historic structures and prehistoric or historic artifacts are encountered during any activity related to the construction of the material source, the contractor shall immediately cease operations within a 50 foot radius of the discovery location and notify the Engineer.  In the event that an unmarked human burial and/or funerary remains are encountered during any activity related to the construction of the material source, the contractor shall immediately cease operations within a 100 foot radius of the discovery location and notify the Engineer.   The Engineer will contact the Environmental Planning – Historic Preservation Team so that appropriate notification of the discovery is made per state and federal regulations.

After notifying the Engineer, the contractor shall, within good faith, secure the area and take all reasonable measures to protect the historical and cultural resources.  No activity shall resume unless authorized by the Engineer.  The Engineer will not authorize resumption of any activity until receiving confirmation from the Environmental Planning – Historic Preservation Team that the contractor may commence work.

If the Department determines that the proposed use will have major adverse impact on cultural or historic resources, the Department will not allow the use of the source.

- Permit from Navajo Nation:

For projects located on the Navajo Reservation, the Navajo Nation has adopted a permitting system for any sources, regardless of whether on or off the Navajo reservation, which are to supply material for projects located within its boundaries. No material source will be approved until the contractor submits a copy of the permit from the Navajo Nation allowing materials from the proposed source to be used on the project. For information concerning the permit, the contractor shall contact the Navajo Nation Historic Preservation Office.

- Bureau of Land Management Material Sources:

If the contractor elects to pursue the use of material sources on BLM land under Title 30 Code of Federal Regulations, it is at the contractor’s sole risk, and the Department bears no responsibility for any delays or costs associated with the request to use material sources on BLM Land.

The Department will not request or pursue any “free-use permit” under Title 23 Code of Federal Regulations or any other arrangement with BLM on this project.

1001-4	Special Access within Right Of Way:

The contractor may submit a request to the District Engineer to approve special access to a controlled access highway if special access is not shown on the project plans.

The request by the contractor shall be accompanied by an environmental analysis and by documents which specify the point(s) of access, the acquisition of right of way, the manner in which access will be attained, the traffic control plan, and crossovers, along with all other appropriate data which will allow the District Engineer to evaluate its request. If the request is approved, a supplemental agreement shall be entered into.

All costs associated with the special access requested by the contractor shall be borne by the contractor, including, but not limited to, cattle guards, fences, gates and restoration work.

When access is not being utilized, gates shall be closed and locked. Upon completion of all operations, the area within the right of way that has been disturbed shall be restored to the condition existing prior to the contractor's operations.

The decision by the District Engineer to deny a request by the contractor will be considered to be final.

Access to private property shall be maintained to keep inconvenience to the property owner to a minimum. Prior to any construction in front of driveways, the Contractor shall notify the property owner 24 hours in advance. Inconvenience caused by construction across driveways and sidewalks shall be kept to a minimum by restoring the serviceability as soon as possible.

1001-5	Fences and Cattle Guards:

Where the haul roads to material sources cross existing fence lines in areas where there is livestock of any kind, temporary cattle guards shall be installed by the contractor at each crossing.

The livestock operator or owner shall be contacted prior to the beginning of any operations and effective measures shall be taken and means provided by the contractor to prevent livestock from straying.

In operations where conditions will exist that are dangerous to livestock of any kind, temporary cattle guards and fence shall be installed around the pit area by the contractor to protect livestock.

Temporary cattle guards and fence installed by the contractor shall be removed and existing fence disturbed shall be replaced or reconstructed and all fence shall be left in as good condition as it was prior to the beginning of work.

(1005PG, 12/21/23)

SECTION 1005	BITUMINOUS MATERIALS:

1005-3		Bituminous Material Requirements:

1005-3.01		Asphalt Cement: of the Standard Specifications is revised to read:

Asphalt cement shall be a performance grade (PG) asphalt binder conforming to the requirements of AASHTO M 320.  Air blown/oxidized asphalt, Polyphosporic acid (PPA) modification and re-refined/recycled engine oil bottom (REOB) will not be accepted.    The pressure aging temperature for all binders, including Terminal Blend rubberized binder and Polymer modified asphalt binder shall be as specified below:

| PG Asphalt Binder | Pressure Aging Temperature |
| --- | --- |
| PG 70-XX and above | 110 ºC |
| PG 64-XX and below | 100 ºC |

If Terminal Blend rubberized binder (XX-XXTR+) is used, it shall conform to the requirements of Table 1005-1 and 1005-1a.

If Polymer modified asphalt binder (XX-XXPM) is used, it shall conform to the requirements of Table 1005-1 and 1005-1b.

If, during asphaltic concrete production, it is determined by testing that asphalt cement fails to meet the requirements for the specified grade, the asphaltic concrete represented by the corresponding test results shall be evaluated for acceptance.  Should the asphaltic concrete be allowed to remain in place, the contract unit price for asphalt cement will be adjusted by the percentage shown in Table 1005-1.  Should the asphalt cement be in reject status, the contractor may, within 15 days of receiving notice of the reject status, supply an engineering analysis of the expected performance of the asphaltic concrete in which the asphalt cement is incorporated.  The engineering analysis shall detail any proposed corrective action and the anticipated effect of such corrective action on the performance.  Within three working days, the Engineer will determine whether or not to accept the contractor’s proposal.  If the proposal is rejected, the asphaltic concrete shall be removed and replaced with asphaltic concrete meeting the requirements of the specifications at no additional expense to the Department.  If the contractor’s proposal is accepted, the asphaltic concrete shall remain in place at the applicable percent of contract unit price allowed, and any necessary corrective action shall be performed at no additional cost to the Department.

1005-3.03		Emulsified Asphalt: the fourth paragraph of the Standard Specifications is revised to read:

Emulsified asphalts shall be homogeneous.  If emulsified asphalt has separated, it shall be thoroughly mixed to insure homogeneity. If emulsified asphalt has separated due to freezing, it shall not be used.  Emulsified asphalt shall not be used after 30 days from production.

1005-3.04		Emulsified Asphalt (Special Type): of the Standard Specifications is revised to read:

Emulsified asphalt (special type) shall consist of Type SS-1 or CSS-1 diluted with water to provide an asphalt content not less than 26 percent.  The water used shall be potable.  Potable water obtained from public utility distribution lines will be acceptable.  The water used shall be free of injurious amounts of oil, acid, alkali, clay, vegetable matter, silt, or other harmful matter.  The material shall not be diluted in the field.

1005-3.07		Other Requirements: the tables 1005-1a and 1005-1b of the Standard Specifications are revised to read:

| TABLE 1005-1a Terminal Blend rubberized binder (XX-XXTR+) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Property | Test Method | Requirement | Test Result | Percent of Contract Unit Price Allowed |
| Solubility, %, minimum | ASTM D7553 or ASTM 2042 | 98 | ----- | ----- |
| Elastic Recovery, @ 10 ºC, %, minimum | AASHTO T 301 | 75 | 75 70 - 74 < 70 | 100 80 65 (1) |
| - Reject Status: The pay adjustment applies if allowed to remain in place. |  |  |  |  |
| Notes: In case of dispute, ASTM D2042 shall be used to determine the Solubility. The asphalt binder shall contain a minimum of 8 percent crumb rubber and a minimum of 3 percent SBS (styrene-butadiene-styrene) polymer. The crumb rubber shall be derived from processing whole scrap tires or shredded tire materials. The tires from which the crumb rubber is produced shall be taken from automobiles, trucks, or other equipment owned and operated in the United States. The processing shall not produce, as a waste product, casings or other round tire material that can retain moisture when stored or disposed of above ground. Modified binders shall be blended at the source of supply and delivered as a homogenous mixture to the job site. Modified Binders stored at the asphalt concrete mixing plant for more than two weeks or beyond the supplier recommended shelf life, whichever is less, shall be sampled and tested. |  |  |  |  |

| TABLE 1005-1b Polymer Modified Asphalt Binder (XX-XXPM) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Property | Test Method | Requirement | Test Result | Percent of Contract Unit Price Allowed |
| Solubility, %, minimum | ASTM D7553 or ASTM D2042 | 98 | ----- | ------- |
| Elastic Recovery @ 10oC, %, minimum | AASHTO T 301 | 75 | ≥ 75 70 - 74 < 70 | 100 80 65 (1) |
| (1) Reject Status: The pay adjustment applies if allowed to remain in place. |  |  |  |  |
| Notes: In case of dispute, ASTM D2042 shall be used to determine the Solubility. Asphalt binder shall contain a minimum of 3 percent SBS (styrene-butadiene-styrene) polymer. Modified binders shall be blended at the source of supply and delivered as a homogenous mixture to the job site. Modified Binders stored at the asphalt concrete mixing plant for more than two weeks or beyond the supplier recommended shelf life, whichever is less, shall be sampled and tested. |  |  |  |  |

1005-3.07		Other Requirements: the table 1005-1c of the Standard Specifications is hereby deleted.

1005-3.07		Other Requirements: the table 1005-3 of the Standard Specifications is revised to read:

| TABLE 1005-3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EMULSIFIED ASPHALTS |  |  |  |  |  |  |  |
| Tests On Emulsion | Test Method | Requirement |  |  |  |  |  |
|  |  | RS-1 | CRS-1 | RS-2 | CRS-2 | SS-1 | CSS-1 |
| Viscosity: Saybolt Furol, seconds, range 77 F 122 F | AASHTO T 59 | 20-100 | 20-100 | 50-400 | 50-400 | 20-100 | 20-100 |
| Settlement: 5 days, %, maximum | AASHTO T 59 | 5 | 5 | 5 | 5 | 5 | 5 |
| Sieve: Retained on No. 20, %, | AASHTO T 59 (1) | 0.10 | 0.10 | 0.10 |  | 0.10 | 0.10 |
| maximum |  |  |  |  |  |  |  |
| Particle Charge | AASHTOT 59 |  | Pos. |  | Pos. |  | Pos. |
|  |  |  |  |  |  |  | (2) |
| Demulsiability: 35 mL, 0.02 N calcium chloride %, minimum | AASHTO T 59 | 60 |  | 60 |  |  |  |
| Classification: Uncoated particles, %, minimum | Arizona Test Method 502 |  |  |  | 55 |  |  |
| Residue: (3) |  | 55 | 60 | 63 | 65 | 57 | 57 |
| Residue, %, |  |  |  |  |  |  |  |
| minimum (4) |  |  |  |  |  |  |  |
| Notes: |  |  |  |  |  |  |  |
| - Distilled water shall be used. Two percent sodium oleate solution will not be accepted. |  |  |  |  |  |  |  |
| - If the Particle Charge Test result is inconclusive, material having a maximum PH value of 6.7 will be acceptable. |  |  |  |  |  |  |  |
| - Residue will be obtained in accordance with the requirements of Arizona Test Method 504 and shall conform to all the requirements of AASHTO M 320 for PG 64-16, except that for CRS-2 the dynamic shear (G*/Sin ) on the original residue shall be a minimum of 1.00 kPa and a maximum of 1.50 kPa. |  |  |  |  |  |  |  |
| - Residue by evaporation may be determined in accordance with the requirements of Arizona Test Method 512; however, in case of dispute, AASHTO T 59 will be used. |  |  |  |  |  |  |  |

1005-3.07		Other Requirements: the note (2) of Table 1005-3a of the Standard Specifications is revised to read:

- Testing shall be performed on residue by distillation. Testing on residue by oven evaporation will not be accepted.

1005-3.07		Other Requirements: the table 1005-5 of the Standard Specifications is revised to read:

| TABLE 1005-5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| EMULSIFIED RECYCLING AGENTS |  |  |  |  |  |
| Tests on Emulsified Recycling Agent | Test Method | Requirement |  |  |  |
|  |  | ERA-1 | ERA-5 | ERA-25 | ERA-75 |
| Viscosity: Saybolt Furol, 77 F, seconds range | AASHTO T 59 | 15 - 40 | 15 - 100 | 15 - 100 | 15 - 100 |
| Miscibility | AASHTO T 59 | Passes | Passes | Passes | Passes |
| Sieve Test: %, maximum | AASHTO T 59 (1) | 0.10 | 0.10 | 0.10 | 0.10 |
| Particle Charge | AASHTO T 59 | Positive | Positive | Positive | Positive |
| Residue: (2) Residue, %, minimum | (3) | 60 | 60 | 60 | 60 |
| Notes: (1) Distilled water shall be used. Two percent sodium oleate solution will not be accepted. (2) Residue will be obtained in accordance with the requirements of Arizona Test Method 504 and shall conform to the requirements specified in Table 1005-4. |  |  |  |  |  |
| (3) Residue by evaporation may be determined in accordance with the requirements of Arizona Test Method 512; however, in case of dispute, AASHTO T 59 will be used. |  |  |  |  |  |

1005-3.07		Other Requirements: the Paving Asphalt row of the table 1005-6 of the Standard Specifications is modified to add:

| TABLE 1005-6 |  |  |  |  |
| --- | --- | --- | --- | --- |
| OTHER REQUIREMENTS |  |  |  |  |
| Grade of Asphalt Specification Designation | Range of Temperatures for Application by Spraying, F (Not applicable for Plant Mixing) | Range of Aggregate Temperatures for Plant Mixing, F |  | Basis of Conversion, Average Gallons Per Ton at 60 F |
| Paving Asphalt | 275 - 400 | ------ |  |  |
| PG 76-22 PM 229 PG 70-22 PM 230 PG 64-28 PM |  |  |  | 231 232 233 |

(1006PCC, 01/20/22)

SECTION 1006	PORTLAND CEMENT CONCRETE:

1006-2.01(A)	General: of the Standard Specifications is revised to read:

Cementitious material is defined as an inorganic material or a mixture of inorganic materials that sets and develops strength by chemical reaction with water by formation of hydrates and is capable of doing so under water.  In this specification, cementitious materials are defined as hydraulic cement (Portland cement, Portland-pozzolan cement or Portland-limestone cement) and supplementary cementitious materials (fly ash, natural pozzolan, or silica fume).

1006-2.01(B)	Hydraulic Cement: the first paragraph of the Standard Specifications is revised to read:

Hydraulic cement shall consist of Portland cement, Portland-pozzolan cement or Portland-limestone cement.

1006-2.01(B)	Hydraulic Cement: of the Standard Specifications is modified to add:

Portland-limestone cement shall conform to the requirements of ASTM C595 for blended hydraulic cement with moderate sulfate resistance, Type IL (MS).

1006-2.01(C)(2)	Flyash and Natural Pozzolan: of the Standard Specifications is revised to read:

Fly ash and natural pozzolan shall conform to the requirements of ASTM C618 for Class C, F, or N.

TABLE 1006-5	Design Criteria: the column heading of the third column of the Standard Specifications is revised to read:

|  |  | Cementitious Material Content: Lbs. per Cu Yd. Minimum - Maximum |  |  |
| --- | --- | --- | --- | --- |

1006-3.02(C)	Mix Design Submittal Requirements: the items (16) and (18) of the Standard Specifications are revised to read:

- Volume of each material measured to the nearest hundredth of a cubic foot;

- Total volume measured to the nearest hundredth of a cubic foot;

If the Engineer determines that the final contract amount earned will fall between 75 and 100 percent of the original contract amount, lump sum traffic control will be compensated at 100 percent once final acceptance is achieved.

In the event the Town revises the contract in accordance with Subsection 104.02 of the specifications due to a significant change in the character of work, and the revisions impact the Maintenance and Protection of Traffic, the lump sum price for Maintenance and Protection of Traffic will be adjusted as part of the contract revision under the relevant supplemental agreements.

No adjustments to the lump sum price for Maintenance and Protection of Traffic will be made for suspension of work for seasonal restrictions.

Revisions in the phasing of construction or maintenance of traffic control operations, requested by the contractor, may require traffic control to be installed in accordance with standards and/or designs other than those included in the plans. Such revisions or modifications to the traffic control plans shown in the contract shall be submitted by the contractor for approval by the Engineer. No additional payment will be made for the revised plans requested by the contractor.

701-6	Basis of Payment for Elements of Work (In Use): of the ADOT Standard Specifications is hereby deleted.

# SECTION 810	EROSION CONTROL AND POLLUTION PREVENTION

  - Method of Measurement: of the ADOT Standard Specification is revised to read:

The Town will reimburse the contractor for erosion control on a lump sum basis. No additional measurement for payment to the contractor will be made for this work.

Following the initial installation of the erosion control, the Engineer may direct the contractor to move the Element of Work from one location and reinstall it at another location.

Any deficiencies in the erosion control plan, devices, equipment, services, or other elements of work will be brought to the attention of the contractor by the Engineer and all deficiencies shall be corrected before the close of that work shift, unless otherwise specified. The contractor shall not be paid for those deficient Elements of Work unless restored to full usefulness prior to the close of the work shift in which notice of the defect is given. Measurement for payment will not resume until the beginning of the work shift following that work shift in which those elements are restored to usefulness.

Measurement for payment of the Elements of Work will begin on the day they are installed in place for erosion control.

  - Basis of Payment: of the ADOT Standard Specification is revised to read:

The accepted quantities of erosion control, measured as provided above, will be paid for as lump sum, which price shall be full compensation for all labor, materials, tools, stakes, equipment, and incidentals necessary for furnishing and installing the devices, complete in place, as approved by the Engineer. The work includes excavation, preparation, compaction, installation, maintenance, removal of sediment debris, removal and disposal of elements, developing the SWPPP, and any revisions to the erosion control plan.